Wolbachia DNA Extraction Protocol: Part I

Place a check mark next to each step after you complete it.

__________ Turn on the incubator (dry bath or hot water bath) and adjust to 56o C.

__________ Put on laboratory gloves. It is important to use sterile technique throughout this lab activity.

|  | Tube Label | Contents |
| --- | --- | --- |
| 1 |  | Student #1’s arthropod |
| 2 |  | Student #2’s arthropod |
| 3 |  | Positive (+) Arthropod Control |
| 4 |  | Negative (-) Arthropod Control |

__________ Carefully disinfect your lab table surface with 70% ethanol solution and paper towels. Spray your gloves with the 70% ethanol solution.

__________ Using a permanent maker (Sharpie), label four microcentrifuge tubes with your combined initials and sample number 1-4 (for example, Paul and Suzie would be: PS1, PS2, PS3, and PS4). Complete the table for your samples and record in your lab notebook.

SAMPLE PREPARATION

__________ Obtain the (+) and (-) arthropod controls from your instructor.

Instructions continued on next page

| _______ | 1 | If preserved in alcohol, use tweezers to carefully transfer the first collected arthropod to a paper towel. Rinse with water and blot dry excess liquid. |  |
| --- | --- | --- | --- |
| _______ | 2 | Take a picture of your arthropod. |  |
| _______ | 3 | FOR SMALL ARTHROPODS: If the specimen is smaller than a grain of rice, use the entire body. FOR LARGE ARTHROPODS: Use a scalpel or scissors to remove the abdomen and cut off a small piece (roughly ~2 mm). If it has a thick/tough exoskeleton, dissect out the abdominal soft tissue and/or cut into multiple pieces so that the soft tissue is exposed. | abdomen |
| _______ | 4 | Place the specimen in the corresponding labeled microcentrifuge tube. Carefully rinse and clean the dissection tools. | - 1 |
| _______ | 5 | Working with your partner, repeat steps #1-4 for the second arthropod and for the (+) and (-) arthropod controls. Dissections are not necessary for the controls as they are small enough to use the entire body. | 2 3 4 + |

CELL LYSIS

Instructions continued on next page

SAMPLE PREPARATION

*PLEASE NOTE: These techniques are sensitive to errors. If you do not pipette the correct amounts into your tube, you will not get results.  Double check your pipette volumes!

| _______ | 6 | Add 180 µL Buffer ATL to the tube containing your first arthropod and discard the tip. Use a sterile pestle to thoroughly grind the sample for about a minute. Ensure that the pestle is tightly pressed against the tube and apply muscle power to homogenize the sample. Remove the pestle and place on a paper towel. | 0 1 8 1 |
| --- | --- | --- | --- |
| _______ | 7 | Using a new pipette tip and new pestle for each sample, repeat the above step with remaining arthropods. | Change tips 2 1 0 8 4 3 |
| _______ | 8 | Using new pipette tips for each sample, add 20 µL proteinase K to each of the microcentrifuge tubes. | 2 0 0 0 0 2 Change tips |
| _______ | 9 | Using new pipette tips for each sample, add 200 µL Buffer AL to each of the microcentrifuge tubes. | 2 0 0 0 2 0 Change tips |
| _______ | 10 | Immediately mix each tube by vortexing for 10 seconds. | 10 sec |
| _______ | 11 | Place your tubes in the incubator where they will incubate at 56o C for at least 15 minutes. | 15 min 56 C |
| _______ | 12 | While the tubes are incubating, label four new microcentrifuge tubes using the same labels as above. | 4 3 2 1 |
| _______ | 13 | Collect tubes from the incubator and spin in the centrifuge for about 30 seconds at 8000 rpm to pellet debris. Ensure that the rotor is balanced and centrifuge lids are properly secured (if applicable). Confirm with your instructor before your first spin. DNA is now in the supernatant (the clear liquid above the debris pellet). | Supernatant Balance the rotor 8000 rpm 30 sec |

DNA PRECIPITATION

| _______ | 14 | NOTE: This step requires patience and care! Carefully pipette or pour the supernatant from each arthropod tube to the newly labeled microcentrifuge tube with corresponding label. Do not dislodge the pellet. If needed, re-spin the sample and use a P1000 pipette to transfer the supernatant (less than 400 µL; some liquid will remain in the tube). | Do not transfer the pellet |
| --- | --- | --- | --- |
| _______ | 15 | Discard debris pellet tubes and keep the supernatant tubes – this is where your DNA is located. | Discard Keep |
| _______ | 15 | Using new pipette tips for each sample, add 200 µL Ethanol to each of the supernatant-containing microcentrifuge tubes. | 0 0 2 2 0 0 Change tips |
| _______ | 16 | Immediately mix each tube by vortexing for 10 seconds. DNA is now precipitated. | 10 sec |

Optional stopping point. Precipitated DNA may be stored in the refrigerator (4o C).

Wolbachia DNA Extraction Protocol: Part II

Place a check mark next to each step after you complete it.

__________ Put on laboratory gloves (it is important to use sterile technique).

__________ Carefully disinfect your lab table surface with 70% ethanol solution and paper towels. Spray your gloves with the 70% ethanol solution.

DNA PURIFICATION: BINDING DNA TO THE MEMBRANE

| _______ | 1 | Collect four Spin Columns fitted with four Flow Through tubes. Label the lids of the Spin Columns with your sample numbers from Part I. *NOTE: The Spin Column sits inside the Flow Through tubes. DO NOT SEPARATE the Spin Column from the Flow Through tube. | * Pre-assembled in the kit Spin Column |
| --- | --- | --- | --- |
| _______ | 2 | Carefully pour or pipette the liquid from your first sample (total volume ~600 µL; include any solid precipitate at the bottom of the tube) into the corresponding Spin Column. Repeat this process with the three other tubes, keeping the tube numbers consistent. *BE CAREFUL TO NOT PUNCTURE THE MEMBRANE IN THE SPIN COLUMN | 4 1 2 3 Flow Through tube |
| _______ | 3 | Place your tubes in the centrifuge and spin for 1 minute at 8000 rpm. DNA is now bound to the Spin Column membrane. | Balance the rotor 1 min 8000 rpm |
| _______ | 4 | Discard liquid from the bottom of each Flow Through tube onto a paper towel (you will reuse this towel). Place the Spin Columns back in the same Flow Through tubes. |  |

DNA PURIFICATION: WASHING THE DNA

Instructions continued on next page

Change tips

| _______ | 5 | Using new pipette tips for each sample, add 500 µL Buffer AW1 to each Spin Column. Use caution to not touch the white membrane with your pipette tips. | 4 0 5 0 1 2 3 |
| --- | --- | --- | --- |

| _______ | 6 | Place your tubes in the centrifuge and spin for 1 minute at 8000 rpm. | Balance the rotor 1 min 8000 rpm |
| --- | --- | --- | --- |
| _______ | 7 | Discard liquid from the bottom of each Flow Through tube onto a paper towel. Place the Spin Columns back in the same Flow Through tubes and discard the paper towel. |  |
| _______ | 8 | Using new pipette tips for each sample, add 500 µL Buffer AW2 to each Spin Column. Use caution to not touch the white membrane with your pipette tips. | 4 3 2 1 0 5 0 Change tips |
| _______ | 9 | Place your tubes in the centrifuge and spin for 3 minutes at 14,000 rpm. | 3 min 14,000 rpm Balance the rotor |
| _______ | 10 | Discard Flow Through tubes and keep the spin column – this is where your DNA is located. | Discard Keep |

DNA ELUTION

| _______ | 11 | Collect four new microcentrifuge tubes and label the lids AND sides of the tubes with your sample numbers. | 3 2 4 1 1 2 4 3 |
| --- | --- | --- | --- |

Instructions continued on next page

| _______ | 12 | Place your spin columns into the corresponding microcentrifuge tubes (note: you will not be able to close the microcentrifuge tube lid). | 4 3 2 1 |
| --- | --- | --- | --- |
| _______ | 13 | Using new pipette tips for each sample, add 100 µL Buffer AE directly to the white membrane in each spin column. Use caution to not touch the membrane with your pipette tips. | 1 0 0 Change tips |
| _______ | 14 | Let your tubes sit at room temperature for 1 minute. | 1 min |
| _______ | 15 | Place your tubes in the centrifuge and spin for 1 minute at 8000 rpm. This will pull DNA through the Spin Column and into the microcentrifuge tubes. | 1 min 8000 rpm Balance the rotor |
| _______ | 16 | Discard the Spin Columns. Keep the microcentrifuge tubes – this is where the DNA is located. | Keep Discard |
| _______ | 17 | Make sure your final microcentrifuge tubes are clearly labeled both on the top and on the side with your initials and sample number. If any of the lids snapped off during the final spin, transfer DNA to a new labeled tube. | 3 2 4 1 1 2 4 3 |

Illustrations created with BioRender.com

Place DNA tubes in a storage box. Label the box with colored lab tape, create an organized record of the contents, and store the DNA box in the freezer (-20o C).